CRC / 9-bit RS-232, RS-422, RS-485 demo:

This is a demo illustrating (1) 8-bit CRC check sums, (2) 57600,N,9,1

communications, and (3) RS-422/RS-485 communications.

crc.asm ... A program that prints out an Intel Hex Format dump of addresses

0 to 1fffh in code space.  However, instead of calculating check

sums by adding all the bytes on each line, as is done with Intel

Hex Format, an 8-bit CRC is produced.

This also demonstrates communications at 57600 baud using 9 bit

addressing.  The serial address in this program is ':'.  Every

key except 's' is echoed.  A hex dump, as described above, is

sent when 's' is received.  Responses will only be produced

when the address received exactly matches ':'.

crc.c .... A C program that will read an CRC-modified intel hex file and

verify it.

sample.hex.. A sample hex file.

client/demo.c  A 57600,N,9,1 demonstration client.  This client will work on

RS-232, RS-422, and RS-485.

The CRC polynomial used by both programs is:

x^8 + x^6 + x^5 + x^3 + x^2 + x^0.

The following assumptions are made about RS-485 links:

* The T0 line enables the transmit driver at the 8051 when it is set,

and disables the driver when cleared.

* OUT1 of the Modem Control Register similarily drives the TX enable

line on the PC host's RS-485 communications card.