;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

;	INTEL MCS-51 MASTER  CONTROLLER MODULES

;

;	The following ASM51 software emulation modules are used to develop I2C-bus

;	functionality with Intel MCS-51 microcontrollers.  They are described in detail in

;	FaxBACK document #2175 and BBS document AP476.ZIP.

;

;	Written By:  	Sabrina Quarles

;			Intel Corporation

;			EMD 8-Bit Applications Engineering	Rev 1.0

;	Date:		December 1, 1992

;

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

;

;	SEND STOP Subroutine

;

;	This program sends an I2C STOP condition to release the bus.

;

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

SEND_STOP:

CLR     SDA_PIN         	;Get SDA ready for stop.

%RELEASE_SCL_HIGH	;Set clock for stop.

%DELAY_3_CYCLES	;Delay

SETB    SDA_PIN	;Send I2C STOP

;Delay satisfied via software

CLR     I2C_BUSY	;Clear I2C busy status.

RET			;Bus should now be released.

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

;

;	SEND_MSG Subroutine

;

;	This subroutine sends a message across the I2C bus using the

;	information stored in the XMT_DAT Buffer in the following format:

;

;	Buffer @R0 = SlvAddr, # of Bytes to be Transferred, Data Bytes

;

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

SEND_MSG:

MOV SLV_ADDR, @R0	;Initializes Slave Address

INC R0                  	;Next address

MOV BYTE_CNT,@R0	;Initializes BYTE_CNT

INC R0			;Next address

ACALL SEND_DATA	;Send Data

RET			;Return from Subroutine

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

;

;    MASTER CONTROLLER Subroutine

;

;    This subroutine sends an I2C START condition and Slave Address to

;    begin I2C communications.

;

;       SDA = Receive/Transmit Data

;       SCL = Generate/Control Clock Line

;

;

;    Slv_Addr = Slave Address

;

;    Verification

;       Issues before MASTER TRANSMIT

;           * No Bus Fault = Bus Not Busy = SCL & SDA HIGH

;

;

;        Issues during MASTER TRANSMIT

;            *  ACK Received after every Byte Transmission

;

;

;       SUBROUTINES Used

;           Send_Byte

;

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

MASTER_CONTROLLER:

SETB    I2C_BUSY		;Indicate that I2C frame is in progress.

CLR     NO_ACK		;Clear error status flags.

CLR     BUS_FAULT

JNB     SCL_PIN, FAULT		;Check for bus clear.

JNB     SDA_PIN, FAULT

CLR     SDA_PIN		;Begin I2C start.

%DELAY_3_CYCLES		;Delay

CLR     SCL_PIN		;Complete I2C start.

%DELAY_3_CYCLES		;Delay

MOV     A,SLV_ADDR		;Get slave address.

ACALL   SEND_BYTE		;Send slave address.

RET

FAULT:

SETB    BUS_FAULT		;Set fault status

RET				; and return.

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

;     MASTER TRANSMIT ~ SEND_BYTE Subroutine

;

;           This subroutine sends 1 byte of information located in the ACCumulator

;               ACC = Byte to be Transmitted

;

;       Verification Issues

;           *  ACK Received after transmission of Byte

;

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

SEND_BYTE:

MOV     BIT_CNT,  #8		;Set bit count value.

SB_LOOP:

RLC     A			;Send one data bit.

MOV     SDA_PIN,C		;Put data bit on pin.

%RELEASE_SCL_HIGH		;Drives SCL HIGH

%DELAY_3_CYCLES		;Delay

CLR     SCL_PIN

%DELAY_3_CYCLES  		;Delay

DJNZ    BIT_CNT,SB_LOOP	;Repeat until all bits sent.

SETB    SDA_PIN		;Release data line for acknowledge.

%RELEASE_SCL_HIGH		;Send clock for acknowledge.

%DELAY_4_CYCLES		;Delay

JNB     SDA_PIN, SB_EX		;Check for valid acknowledge bit.

SETB    NO_ACK		;Set status for no acknowledge.

SB_EX:

CLR     SCL_PIN		;Finish acknowledge bit.

%DELAY_3_CYCLES		;Delay

RET				;Return

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

;

;   MASTER TRANSMIT ~ SEND DATA Subroutine

;

;   This subroutine transmits multiple data bytes over the SDA line.

;   The following locations must be initialized before the transmission.

;

;       Byte_Cntr = # of bytes to be transmitted

;       Slv_Adr   = Slave Address

;       @R0       = Data to be Transmitted

;                       ~ includes any addition subaddresses, control, etc

;                           specific to certain devices

;

;       SUBROUTINES Used

;           MASTER_XMIT

;               Send_Byte

;           Send_Byte

;

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

SEND_DATA:

ACALL   MASTER_CONTROLLER	;Acquire bus and send slave address.

JB      NO_ACK,SDEX			;Check for slave not responding.

SD_LOOP:

MOV     A,@R0				;Get data byte from buffer.

ACALL   SEND_BYTE			;Send next data byte.

INC     R0				;Advance buffer pointer.

JB      NO_ACK,SDEX			;Check for slave not responding.

DJNZ    BYTE_CNT, SD_LOOP		;All bytes sent?

SDEX:

ACALL   SEND_STOP	;Done, send an I2C stop.

RET

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

;

;	TRANSFER Subroutine

;

;	This subroutine copies data from the EPROM referenced by DPTR into a

;   	Buffer referenced by R1.

;

;	DPTR = String stored into EPROM

;	R1 = Buffer in which data shall be stored

;

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

TRANSFER:

CLR A			;Clears ACC

MOVC A, @A+DPTR	;Moves contents of DPTR into A

MOV @R1, A		;Copies A into Buffer

INC R1			;Next address

INC DPTR		;Next location

CLR A			;Clears ACC

MOVC A, @A+DPTR	;Moves contents of DPTR into A

MOV @R1, A		;Copies A into Buffer

MOV R0, A		;Copies A into R0 (# of bytes)

INC R1			;Next address

INC DPTR		;Next location

CLR A			;Clears A

NEXT:

MOVC A, @A+DPTR	;Moves contents of DPTR into A

DEC R0			;Decrease # of remaining bytes

MOV @R1, A		;Copies A  into Buffer

INC R1			;Next address

INC DPTR		;Next location

CLR A			;Clears A

CJNE R0, #0, NEXT	;Compare # of bytes remaining

RET			;If all bytes copied, return

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

;	RECV_MSG Subroutine

;

;	This subroutine receives a message from the I2C bus using SLV_ADDR

;	and BYTE_CNT as indicators as to what Slave will be sending info and

;	how many bytes to expect to receive, and places the data into the

;	RCV_DAT buffer.  The RCV_DAT Buffer is configured to receive a max. 8 bytes.

;

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

RECV_MSG:

MOV SLV_ADDR, @R1	;Moves SLV_ADDR from Buffer R0 points to

INC R1			;Next buffer location

MOV BYTE_CNT, @R1	;Moves BYTE_CNT value into memory location

ACALL RCV_DATA	;Calls Rcv Data Subroutine

RET			;Returns from Receive Msg subroutine

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

;

;	MASTER RECEIVE ~ RECEIVE BYTE Subroutine

;

;	This subroutine receives a byte from an addressed I2C slave

;	device and places into the ACC register.

;

;	ACC = Data Byte Received

;

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

RECV_BYTE:

MOV     BIT_CNT,#8	;Set bit count.

RB_LOOP:

%RELEASE_SCL_HIGH		;Read one data bit.

%DELAY_3_CYCLES  		;Delay

MOV     C,  SDA_PIN		;Get data bit from pin.

RLC     A			;Rotate bit into result byte.

CLR     SCL_PIN		;Clear SCL pin

%DELAY_3_CYCLES         	;Delay

DJNZ    BIT_CNT, RB_LOOP	;Repeat until all bits received.

PUSH    ACC			;Save accumulator

MOV     A, BYTE_CNT		;Copies byte count into A

CJNE    A, #1, RB_ACK		;Check for last byte of frame.

SETB    SDA_PIN		;Send no acknowledge on last byte.

SJMP    RB_ACLK		;No ACK on last byte; jump to RB_ACLK

RB_ACK:

CLR     SDA_PIN		;Send acknowledge bit.

RB_ACLK:

RELEASE_SCL_HIGH		;Send acknowledge clock.

POP     ACC			;Restore accumulator

%DELAY_3_CYCLES		;Delay

CLR     SCL_PIN		;Clear SCL pin

SETB    SDA_PIN		;Clear acknowledge bit.

%DELAY_4_CYCLES		;Delay

RET				;Return from RecvByte

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

;

;	MASTER RECEIVE ~ RECEIVE DATA BYTES Subroutine

;

;	This subroutine receives multiple data bytes from an addressed

;	I2C slave device into the buffer pointed to by R0.

;

;	BYTE_CNT = # of bytes to be received

;	SLV_ADDR  = Slave addres

;

;	@R0 = location of received data

;

;	SUBROUTINES Used

;		MASTER_XMIT

;		RCV_BYTE

;

;	Note:  To receive with a subaddress, use Send Data to set the

;		subaddress first (no provision for repeated start).

;

;~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

RCV_DATA:

INC  SLV_ADDR			;Set for READ of slave.

ACALL   MASTER_CONTROLLER	;Acquire bus and send slave address.

JB      NO_ACK,RDEX			;Check for slave not responding.

RDLoop:

ACALL   RECV_BYTE		;Recieve next data byte.

MOV     @R0,A			;Save data byte in buffer.

INC     R0			;Advance buffer pointer.

DJNZ    BYTE_CNT,RDLoop	;Repeat untill all bytes received.

RDEX:

ACALL   SEND_STOP	;Done, send an I2C stop.

RET			;Return from RCV_DATA Subroutine

;*************** INTEL CORPORATION ********************

;   ************    I2C MACROS     *****************

;           **********    12/01/92       ***************

;

;

%*DEFINE(Delay_2_Cycles)(

NOP

NOP

)

%DEFINE(Delay_3_Cycles)(

NOP

NOP

NOP

)

%*DEFINE(Delay_4_Cycles)(

NOP

NOP

NOP

NOP

)

%*DEFINE(Delay_5_Cycles)(

NOP

NOP

NOP

NOP

NOP

)

%*DEFINE(Delay_6_Cycles)(

NOP

NOP

NOP

NOP

NOP

NOP

)

%*DEFINE(Delay_7_Cycles)(

NOP

NOP

NOP

NOP

NOP

NOP

NOP

)

%*DEFINE(Delay_8_Cycles)(

NOP

NOP

NOP

NOP

NOP

NOP

NOP

NOP

)

%*DEFINE(Release_SCL_High)(

SETB    SCL_Pin

JNB     SCL_Pin, $

)