***********************************************************************

MicroSim(TM) Corporation

Evaluation Software

***********************************************************************

The evaluation versions of MicroSim Schematics and MicroSim PSpice A/D

allow you to set up circuits and simulations for analog, digital, and

mixed analog/digital designs.

These device limitations apply:

- 64 analog nodes

- or, 10 transistors (any combinationn of B, M, Q, or J devices)

- or, 2 opamps

- or, 10 transmission lines (up to 4 coupled)

- or, 65 digital primitive devices

- or, logic output transitions limited to 10000

- or, logic expression primitives limited to 36 I/O pins

- or, any combination of the above (which will result in a

lower allowable number of each)

Additional limitations include:

- device characterization for diodes only

- stimulus generation for sine waves only

- libraries with approximately 22 analog and 140 digital parts

- synthesis of up to 3rd order filters

Schematic Editor limitations include:

- 1 schematic page (unlimited hierarchy)

- up to 25 parts on a page

- A-size page only

- up to 20 user-defined symbols

- no printing from within the Symbol Editor

- no export/import of symbols

- number of Symbol Library files that can be loaded is limited

to the total number shipped plus one

Netlister limitations include:

- up to 70 real devices for PSpice A/D netlists

- up to 50 symbols, before packaging, for PCB layout netlists

***********************************************************************

New Circuit Analysis Features for Version 6.2

***********************************************************************

Note: If you purchased one of the Design Center/AMD packages, you

need to refer to the file README.TXT which includes information

specific to these programs.  The file README.TXT can be found on the

CD-ROM in the \dctramd\standard or \dctramd\advanced directories (depending

on which package purchased).  After installation this file will reside

in your installation directory.

MicroSim PLSyn users need to refer to the file README.PLS, in addition to

this readme file, for new information specific to MicrSim PLSyn.

This can be found on the CD-ROM in the \msim62 directory.  After

installation this file will reside in your installation directory.

**********************************************************************

The following list highlights the major changes incorporated into the

version 6.2 release.

***** MicroSim PSpice *****

1. Now, initial conditions set on a capacitor or inductor by giving its

IC= attribute a value are automatically processed by PSpice, whether or

not SKIPBP is specified on the .TRAN statement.

This is a change from the default behavior in previous releases where the

IC= attribute was ignored unless the bias point was skipped (i.e. when UIC

was specified on the .TRAN statement).  The NOICTRANSLATE option is provided

if you wish to revert to the older behavior.

2. Time derivative and integral functions are now available for use

with Analog Behavioral Modeling.  These are:

DDT(x)		Time derivative of x

SDT(x)		Time integral of x

These functions enable you to model such things as voltage-dependent

capacitors or to set up systems of coupled differential equations.

(Note that the description of SDT(x) in the features list in the first

chapter of the Circuit Analysis User's Guide is stated incorrectly.)

3. The new default for magnetic CORE models is the improved Level 2 model.

4. A Level 6 BSIM3 model, Berkeley version 2.0, has been added for MOSFETs.

This version adds several new parameters to improve the model's accuracy

over wider ranges of biases and device dimensions.  The Berkeley version 1.0

BSIM3 model continues to be available as Level 5.

5. A Level 4 model has been added for GaAsFETs.  This is the Parker-Skellern

MESFET model (developed at Macquarie University in Sydney Australia) which

provides accurate description of GaAs MESFETs in large-signal RF applications.

6. New vendor-supported models for instrumentation amplifiers, voltage

references, wideband buffers, a current-feedback amplifier, a four-quadrant

analog multiplier, and many other operational amplifiers.

***** MicroSim Probe *****

1. A new search command condition called Point is now provided to allow you

to move to the next data point on the trace (in the same direction as the

last cursor move).

2. The Available Sections dialog now defaults to having all sections selected

(in previous versions, none were selected).

3. Very large trace expressions can now be handled when macros are expanded.

The limit is 10,000 characters.

4. A toolbar is now available.  It provides a shortcut to certain menu and

dialog box items.  The toolbar display can be turned on or off by using

Tools/Options and checking/unchecking Display Toolbar.  The toolbar may

contain more buttons than will show on your screen.  The toolbar can be

only one row high and does not "wrap".  Instead buttons on the right

side of the toolbar may be "truncated" on small screens and/or reduced windows.

The whole toolbar will show on a full size window on a 1024x768 supervga

display.

Note that the Print toolbar button does an immediate print of one copy of the

selected plot window, it does not bring up the print dialog.  Other parameters

of the print, are taken from the values last set in the page setup dialog

(such as margin, cursor box location, portrait/landscape, etc.).

5. Goal Functions, Macros, and Displays are now all contained in one file

called <filename>.prb.

A ".prb" file is an ASCII file which contains three sections, one for display

definitions, one for macros, and one for goal functions.  Each section begins

with a header.  A ".prb" file looks like:

[DISPLAYS]

* Displays go here

[MACROS]

* Macros go here

[GOAL FUNCTIONS]

* Goal functions go here

A ".prb" file can contain any number of display, macro, or goal function

definitions.  The file "msim.prb" is provided at installation and contains many

examples of goal function definitions. This file is located in the directory

specified as PRBFILE in the [PROBE] section of msim.ini.

Two ".prb" files are automatically loaded when a Probe data file is opened: a

global file and a local file.  The global ".prb" file is specified by the

PRBFILE= parameter.  The local file is located in the same directory as the

data file, and has the same name as the data file, but with a ".prb" extension.

For example, a local ".prb" file for the data file "ACdata.dat" would have the

name "ACdata.prb," and would reside in the same directory as "ACdata.dat."

If a local ".prb" file (e.g. "ACdata.prb") does not exist, Probe will search for

old "probe.dsp", "probe.mac" and "probe.gf" files and ask if the user wants

to convert them to a local ".prb" file.  We strongly recommend that the user

accept this automatic conversion.  Old ".mac," ".dsp," and ".gf" files are not

otherwise read in.

The old command line options "-d displayfile," "-m macrofile," and

"-g goalfunctionfile" are no longer supported.  There is a new option

"-p prbfile," which causes Probe to read in the specified ".prb" file after both

the global and the local ".prb" files.

There are two additional ways of creating ".prb" files from existing Display,

Macro, or Goal Function files (if you choose not to let Probe do the

conversion for you automatically):

a)  Use the batch file "cnvt2prb.bat" to convert a ".dsp" and/or

a ".mac" and/or a ".gf" file to a ".prb" file.  (Type cnvt2prb with

no arguments for a complete explanation of the command syntax.)

b)  Use a text editor to add headers to current files, or copy Display,

Macro, or Goal Function file contents to an existing ".prb" file.

Note that any displays, macros, or goal functions being read in from a ".prb"

file will overwrite items of the same type and name.

****** OrCAD Translator ********

Things to know:

- The schematic file, all libraries, and a .cfg file are required to

translate successfully.  If you are missing some libraries, the schematic

will translate, but you will get error messages about unresolved symbols.

If you are missing a .cfg file, the schematic will not translate at all.

- The first release of the OrCAD translator is not intended to translate

designs for use with PLSyn.  Please call MicroSim Technical Support

for the latest information regarding PLD designs.

- Wires that cross pins exactly at the pin hotspot will be connected

after translation.

- Buses must be labeled, and wires attached to the bus must be

labeled as a subset of the bus. Otherwise the wires will not be

connected to the bus.

- OrCAD "Bus Entry" objects are converted into regular wires.

- When drawing OrCAD "bus rippers" users must use the OrCAD

"Bus Entry" object and not simply draw a normal wire at an angle

to look like the regular bus ripper/entry object. Otherwise the

wire will not be connected to the bus.

- Different MicroSim Schematics symbols are created for each port

size. (Remember that OrCAD can size the port symbols to fit the text.)

- Dashed annotation lines are not included in the translated schematic.

- Text with a vertical orientation is translated into rotated text.

- MicroSim Schematics does not support ports with multiple connections.

These ports will be translated as two ports overlaid with pins facing

opposite directions.

- DeMorgan equivalents are not supported.

- The 'simulation' attributes that are added to symbols are based on

the OrCAD standard libraries. Therefore, pin name references in these

attributes could potentially be incorrect if a different (e.g., user drawn)

symbol is being used by the same name.

- Text sizes on translated schematics/symbols are based on an 8 point

font size.

Known problems:

- The OrCAD Translator does not function under Windows/NT.

- Some rotation options for annotation text are not converted correctly.

- ANSI title block graphics have erroneous lines.

- Symbols can not always be found if they are coming from OrCAD libraries

that utilize the "Library Prefix". An example of this is the TTL.LIB.

- Symbols with identically assigned reference designators cause connection

problems. Only the first symbol is actually "connected" to expected wires.

The successive symbols are visually in place but not connected. This is

shown during an ERC check. This can be fixed by simply deleting the symbol

and then un-deleting it.

- The shape ( <===  ===> ) of OrCAD port symbols is not translated

correctly. All ports end up as rectangles.

- Large schematics (multiple pages, hierarchical) can cause the translator

to hang.

- Currently there is no way for a user to prevent the translator from

attempting to translate sub-levels of a hierarchical design.

- Wire/Bus labels are not always placed on the correct segment of the wire.

(But they are always placed on the correct wire.)

- PKGTYPE is on the symbol definition and not the symbol instance.

- The REFDES attribute is always set to visible.

- The OrCAD feature of pins with number zero not being drawn is

not supported. Therefore some symbols are drawn with unexpected

pins or with missing pins. Only the first (GATE A) representation

of the symbol is translated.

- Fill patterns used in symbols are not translated.

- The translator reports "error in configuration file" if the first

line does not read:

{ OrCAD/SDT IV  Configuration File }

This is the string from older versions of OrCAD Draft. So the newer

versions with a first line in the sdt.cfg file of:

{ OrCAD/SDT 386+  Configuration File }

will cause the translator to produce this error. This can error can

be ignored.

Distributed Files:

orctrans.exe    - stand alone version of translator (for larger designs)

package.lst     - a list of package types that is displayed

in the 'Package Type' dialog

orc_map.txt     - A VERY LARGE file that contains default

package types and attributes for symbols

prefix.map      - OrCAD library prefixes that could

potentially be used

orc2edi.cfg     - translator configuration file. (Not to be

altered.)

or2msim.dll     - DLL support file

edi16i.dll      - DLL support file

msimmisc.dll    - DLL support file

Devmap.ini      - OrCAD part field mapping file

If you find additional problems with the OrCAD translator, please contact

Technical Support.  It is our intention to make future revisions of the

translator available to all maintenance customers.

***** MicroSim Schematics *****

1. New Menu Command

FILE/REPORTS:  This new menu entry will allow you to create a bill of materials

report for your schematic.

2. Menu Changes

- The Annotate command has been moved from the analysis menu to the tools menu.

- The Analysis menu has been re-arranged slightly to be more logically

organized.

3. Wiring Changes

In 6.2, wiring has been changed so that ending a segment on a pin or another

wire will terminate the wire but leave you in wire drawing mode ready to

start a new wire.  A snap-to-pin feature has also been added so that if a

segment is ended within a certain distance (user-configurable) of a pin, the

segment will "jump" to that pin.

4. Netlisting - Assignment of node names

When assigning node names to nets with global ports, the netlister now uses

the following rules:

- If the net is attached to AGND, it is named "0"

- If the net is attached to one or more global ports starting with a "$", it

uses the first one alphabetically

- If the net is attached to one or more global ports not starting with a "$",

it uses the first one alphabetically.

5. Netlisting - HI and LO ($D_HI and $D_LO)

The netlister has been changed to treat the HI and LO ports as stimulus.

Previously they were handled like any other global port.  Thus, all nets with

a HI or LO attached ended up as a single net either ($D_HI and $D_LO) to the

netlister.  Now, nets with HI or LO are not combined thus lifting the previous

restriction that layout netlists could not be created for schematics using

HI/LO.  This also corrects related problems with MicroSim PLSyn schematics

using HI/LO.  PSpice netlists are not affected - they use the $D_HI and $D_LO

special node names for any net connected to HI/LO just as before.

6. File/Get Part/Browse

For symbols which have no corresponding simulation model available, a message

to this effect will be displayed underneath the part description for the

currently selected part.

7. Prior to 6.2, Schematics looked in the [MICROSIM] section for POLARIS=ON,

PLSYN=ON and PARAGON=ON to determine whether to display the Run... menu items.

For 6.2, Schematics now looks in the [MICROSIM OPTIONS] section (and

PARAGON= is now OPTIMIZER=).

***** Symbol Editor *****

1. Menu Changes

- Part/Save Changes has been removed having been deemed unnecessary. To save

changes, select File/Save and this will save the part you are editing as well

as the library.  Previously, Part/Save changes would save the changes to the

part but not save them to the library until a File/Save was done.

- File/Return to Schematic has been removed since it was doing the same thing

as File/Close from the symbol editor.

- Window/New Schematic Window was added to allow you to more intuitively get

back to the Schematic editor after having shut down all windows except for the

symbol editor window.

2. Package Editing

Package definitions no longer have attributes since they were not being used.

The new bill of materials instead supports having package "attributes" in a

separate text file.  This text file can be created manually or be created by

exporting information from an outside database.  See documentation for more

details.

3. -sym

Starting the symbol editor by using -sym on the command line now disallows you

from pushing into schematics for hierarchical symbols. The intent of using

-sym is to use the symbol editor in stand-alone mode with no loading of

libraries, etc.

************ MicroSim Parts **********

The Create, Get, and Save toolbar buttons in MicroSim Parts will perform

Create/Part, Get/Part, and Save/part.  Use these actions

for part models.

************ Stimulus Editor *************

The Edit/Edit menu item in the Stimulus Editor is reserved for future

use.  In the current release, it is grayed out.

********** PSpice Optimizer **************

Before starting and optimization, you need to unload the simulation

data from Probe.

******** On-Line Manuals *******

The CD-ROM version of the Windows and Sun MicroSim programs are shipped

with on-line manuals for viewing.  The manuals can be viewed using

FrameViewer which is provided at installation.  Double-click on the

"On-line" manual icon to bring up the MicroSim "Begin" screen.  You

can select which manual to view by clicking once on a manual title.

It is important to note that the manuals are electronic copies of

the hard copy versions.  The manuals are not context-sensitive when

used in concurrence with MicroSim's programs.

Zooming and Window Resizing

The on-line manuals have fixed page sizes.  You can zoom up and down

to see graphics more clearly, but it is best to view the documents at

80% zoom level.  Once set to 80% (from bottom status bar) use

Fit Window to Page (also from bottom status bar) to bring manuals back

to default size.

Printing On-Line Manuals

Using the File/Print command in FrameViewer will print pages exactly

as they are shown on the screen.  There is not a one-to-one page

correspondence for page layout between the on-line and hard copy manuals.

Searching Text

When using the Edit/Find command in FrameViewer to search text strings,

it is important to note that you are searching the current chapter only.

Text will be searched in the active window only.  Use the Table of Contents

and the Index of a manual to find subjects within a whole manual.  You

can search text strings in the Index chapter as well.

***********************************************************************

Features or modifications that did not make it into the 6.2 manuals

***********************************************************************

Installation Note

The Installation Manual states that the Hapserv files are on Disk One

of the installation diskettes.  This is incorrect.  The Hapserv files

can be found on the supplemental network files disk.

***********************************************************************

Caution to users selecting CSDF output from MicroSim example schematics:

There is a readme symbol on all of our shipped example schematics.

This symbol references a readme file which explains information about

the example.  The name of the readme file is <schematic name>.txt.

If you select CSDF output for Probe (Analysis/Probe Setup... Text Data

File Format (CSDF)), then the readme file will be overwritten by the

CSDF file which will have the same name (i.e., <schematic name>.txt.

You can get around this problem by renaming the shipped <schematic name>.txt

file to <schematic name>.rdm (for example) and editing the filename

attribute on the readme symbol to reflect this name change.

***********************************************************************

Library additions:

1)   Added over 120 new models, including a new vendor model library

of pressure sensors from Motorola.  Others include:

*   Analog Devices:  voltage references, instrumentation

amplifiers, low-noise operational

amplifiers,

*   Burr-Brown:  instrumentation amplifiers,

*   Comlinear:  wideband operational amplifiers,

*   Elantec:  video operational amplifiers,

*   National Semiconductor:  high-speed operational amplifiers,

low-voltage operational

amplifiers,

*   Texas Instruments:   low-noise operational amplifiers

2)   All base and copyright parts are now hidden in the symbol libraries.

***********************************************************************

Library List Files

List files of all the MicroSim library devices can be found on the

CD-ROM in the /msim62/lib directory.  These .lis files are text files

that contain listings of the generic device names, model names,

symbol names, and their corresponding .lib and .slb files.  The

library list files are available for browsing and searching on a

text editor.  You can also find these files on the MicroSim BBS.

These files consist of;

List file name           Description of contents

---------------          -----------------------

analog.lis               list of the analog devices (excluding the

European and Japanese analog devices)

dig_ad.lis               list of miscellaneous digital files which

include CMOS, ECL, PAL, GAL, and mixed A/D

interface devices

ttl.lis		  TTL (AC, ACT, ALS, AS, F, H, HC, HCT,

L, LS, S) family of devices

europe.lis               list of the European analog devices

japan.lis                list of the Japanese analog devices

symb_ana.lis             list of all of the analog symbols with

no model definitions

symb_dig.lis             list of all the digital devices with no

model definitions

symb_ad.lis              list of the analog/digital symbols with no

model definitions

***********************************************************************

Bipolar Transistor Model:

A new model parameter XCJC2 has been added to the bipolar transistor model

(Q device).  Its usage is similar to the existing model parameter XCJC.  XCJC

specifies the distribution of CJC capacitance between two branches -intrinsic

base-to-intrinsic collector branch to be XCJC*CJC and extrinsic base-to-

intrinsic collector branch to be (1.0 - XCJC)*CJC.  XCJC2, on the other hand,

specifies the distribution of CJC capacitance between two branches - intrinsic

base-to-intrinsic collector branch to be XCJC2*CJC and extrinsic base-to-

extrinsic collector branch to be (1.0 - XCJC2)*CJC.

The default value for XCJC2 is 1.0.  When 0.0 < XCJC2 < 1.0 is specified, XCJC

is ignored, also values of Cbx2 and Ft2 will be included in the .OP information

to reflect the added branch capacitance and its impact on the Ft.  For backward

compatibility, parameter XCJC and calculation of Cbx and Ft in the .OP information

remain unchanged.

The above changes will produce more accurate results based on the face that the

fraction of CJC modeled inside Rc should be the same fraction of emitter area

to the overall base area.  As a result of this added parameter, better matching

between PSpice-calculated Ft2 and measurement date can be obtained.

***********************************************************************

Note for network license customers only: In the 6.2 release, the file name

"hsrvwin.exe" has been changed to "nhsrvwin.exe."

***********************************************************************

Part and Net Attribute Changes in Schematics:

The names for the part and net attributes which can be

defined for board layout with MicroSim PCBoards have been changed as

follows:

PCBLocationX is now called COMP_X.

PCBLocationY is now called COMP_Y.

PCBRotation is now called COMP_ANGLE.

PCBFixed is now called COMP_FIXED.

DefaultTraceWidth is now called NET_TRACE_WIDTH.

DefaultClearance is now called NET_CLEARANCE.

RoutingStyle is now called NET_ROUTING_STYLE.

Additional attributes that are now available are:

COMP_LAYER to specify the surface of the board on which the

component is to be positioned.

NET_PADSTACK to specify the padstack definition to be used with traces.

******************************************************************************

****** 6.2a Changes to MicroSim Schematics *******

The following is the list of changes in version 6.2a MicroSim

Schematics:

1)  Fixes related to setting up of MicroSim Polaris (Signal Integrity)

analysis.

2)  EDIF layout netlist now will use edif.xnt for net name mapping if it

exists

3)  Edit/Model/Change Model Reference

Fixes a problem that occured when the model name was changed back

to its original value as defined on the symbol.  A Win32s general

protection fault would occur if you later used Edit/Attributes on that

part.

4)  Restricted Operations/Connectivity via Labels

The ERC check no longer issues "unconnected pin" errors if a labeled

wire is dangling but there is another labeled wire of the same name

elsewhere on the page.

5)  Fixes an "Internal Corruption" error that would occur when a schematic

with an unlabeled offpage port connected to a labeled bus was saved.