*******************************************************************************

MicroSim(TM) Corporation

Release Notes for PLSyn Evaluation Version 6.2

*******************************************************************************

------------

INSTALLATION

------------

Please refer to the MicroSim Installation Manual for software installation

instructions.  Package descriptions and tutorials are contained in the PLD

Design System User's Guide.

------------

NEW FEATURES

------------

1. The evaluation version now supports the MACH231 device

2. Toolbars are now supported in Schematics and Probe

3. Importing ABEL and EDIF files in PLSyn

Plsyn is now able to import ABEL and EDIF design files via a new command:

File/Import.  These files are converted to DSL, and in addition can be

converted to a Schematic with a DSL block.  See the on-line help for

more information.

4. LOW_TRUE support

PLSyn now supports LOW_TRUE inputs and outputs.  You can designate a design

input or output signal as active-low by attaching a LOW_TRUE port (in

dig_prim.slb) to the wire.

5. On-Line Help

The on-line help includes several new topics, including:

- System-level signals

- EDIF-to-DSL translation

- DSL library files (.inf,.dsl)

- ABEL translation

- Reference list of programmable logic symbols

6. Choice of Editors for DSL

You can now use a different editor to enter source code for DSL blocks from

within Schematics.  To do this, add the following entry to the [SCHEMATICS]

section of msim.ini:

USEDSLEDITOR=OFF

This causes the standard editor (configured via the EDITOR entry in the

[MICROSIM] section of msim.ini) to be used.  The default standard editor is

NOTEPAD.

7. Low-power Setting on MACHxx1 Devices

You can now use the low-power (low-speed) capability of the AMD MACHxx1

devices (e.g. MACH231), by using the LOW_POWER .pi property. See the on-line

help for the PIL (Physical Information Language) for more information.

---------------------

DOCUMENTATION CHANGES

---------------------

Please make the following changes to the "MicroSim/AMD PLD Design

System User's Guide"

On page 11, near the bottom of the page, add the sentence "Click

right to end." to the end of instruction "8".

On page 21, in the third paragraph, the second sentence should read

"By default, solutions are sorted by pin count."

On page 22, before the last paragraph, insert the following

paragraph:

A message will appear saying "Warning: No test vectors will be

included in the fusemap file...Do you want to continue?".  This

message indicates that the current implementation has not been

simulated in order to produce JEDEC test vectors.  Thus they won't

be included in the resulting JEDEC file.  You can press "Yes" in

response to this warning.